AudioBus FD16
Implementation Brief for Cline / VS Code

16 IN + 16 OUT | 48 kHz | 32-bit | 50 m Cat6A | SN65LVDS049

Purpose: provide an executable technical brief for an AI coding agent to inspect the existing AudioBus repository and implement a new full-duplex FD16 PHY/profile without breaking existing PHYs.

# 0. Instruction to Cline: operating mode

Treat this document as a design specification and implementation constraint set, not as permission to guess. Before editing any file, inspect the current repository state, build system, ESP-IDF version, public APIs, PHY interfaces, and existing tests. Do not overwrite the existing PHYs; add a new FD16 path and keep legacy configurations buildable unless inspection proves otherwise.

Create a written implementation plan first: files to change, interfaces, data flow, clocking, DMA/PARLIO setup, tests, and risk items.

Only after the plan is internally consistent, implement in small compilable steps.

After each meaningful code change, run formatting/build/static checks available in the repository.

Do not invent ESP-IDF APIs: verify exact API names and struct fields against the installed IDF used by the project.

Do not silently change the public AudioBus protocol for legacy modes.

Keep all timing/bit-rate constants derived from named constants, not magic numbers.

# 1. Source of truth and repository inspection

Primary source repository:

GitHub: https://github.com/DatanoiseTV/AudioBus

The repository currently describes AudioBus as a multi-channel TDM audio link over LVDS, with a 1-chip SN65LVDT41 option at 98.3 Mb/s and a 2-chip serializer/deserializer option at 491.5 Mb/s. It states that the existing protocol is half-duplex, uses 8b10b, and sends one frame per 48-kHz sample period. These existing properties are the baseline to preserve for legacy modes, not the target behavior for FD16.

The current GitHub page is the authoritative starting point for file names and architecture; Cline must re-check exact file paths and interfaces from the checked-out revision before applying changes.

# 2. Target product definition

| Parameter | Target |
| --- | --- |
| Nodes | Point-to-point A <-> B |
| Audio IN per node | 16 channels |
| Audio OUT per node | 16 channels |
| Sample rate | 48,000 Hz |
| Sample format | signed 32-bit samples |
| Useful payload / direction | 16 x 32 x 48 kHz = 24.576 Mb/s |
| Transport frame | 72 bytes / sample period |
| Transport line rate | 27.648 Mb/s |
| Transport clock | 27.648 MHz, free-running |
| Coding | NRZ, no 8b10b in FD16 |
| Duplex | Full duplex, independent simultaneous TX/RX |
| PHY | 1 x SN65LVDS049 per endpoint |
| Cable | Cat6A, 4 twisted pairs |
| Target distance | 50 m |
| Primary safety net | CRC + sequence checking + repeat-last-sample concealment |

# 3. Non-negotiable architecture

CAT6A 4 pairs / 50 m

A                                                       B
+------------------+                            +------------------+
| ESP32-P4         |                            | ESP32-P4         |
|                  |                            |                  |
| PARLIO TX        |-- DATA A -> B ----------->| PARLIO RX        |
| PARLIO RX        |<--------- DATA B -> A ----| PARLIO TX        |
|                  |                            |                  |
| local master clk |-- CLOCK A -> B ---------->| external RX clk  |
+--------+---------+                            +---------+--------+
         |                                                |
   SN65LVDS049                                      SN65LVDS049

Use the four cable pairs as follows:

Pair 1: DATA A -> B, LVDS differential.

Pair 2: DATA B -> A, LVDS differential.

Pair 3: CLOCK A -> B, LVDS differential.

Pair 4: controlled reference / shield strategy; do not use it as a power feed.

A is the sole transport-clock master. B recovers no clock from the data stream; it receives the dedicated LVDS clock and uses it as the external PARLIO clock. This is deliberate: it removes the need for a SerDes CDR and avoids an independent clock-frequency mismatch between the two endpoints.

# 4. Why 27.648 MHz / 27.648 Mb/s

16 channels x 32 bits = 512 audio bits/sample

Frame:
  2 B  sync
  2 B  sequence
  1 B  flags
  1 B  channel count
 64 B  audio payload (16 x int32_t)
  2 B  CRC16
------
 72 B total = 576 bits

576 bits/frame x 48,000 frames/s = 27,648,000 bit/s
27.648 MHz / 576 = 48,000 Hz exactly

The exact integer relationship is a design invariant. Do not change frame length, line rate, or sample rate independently without recalculating the relationship.

# 5. FD16 frame format

| Offset | Size | Field | Requirement |
| --- | --- | --- | --- |
| 0 | 1 | SYNC0 | 0xA5 |
| 1 | 1 | SYNC1 | 0x5A |
| 2 | 2 | SEQUENCE | uint16_t, increment every frame, wrap naturally |
| 4 | 1 | FLAGS | 0x22 for 48 kHz / 32-bit / FD16 in first implementation |
| 5 | 1 | CHANNELS | 16 |
| 6 | 64 | AUDIO | 16 x int32_t, deterministic byte order |
| 70 | 2 | CRC16 | CRC-16-CCITT over bytes 0..69 |

Use CRC-16-CCITT with polynomial 0x1021 and initial value 0xFFFF. Use an explicit big-endian wire encoding for each int32_t so the wire format is independent of CPU interpretation.

# 6. Recommended file layout

components/audiobus/
  include/
    ... existing public headers ...
    fd16_frame.h
    ... add FD16 configuration/API only where necessary ...

  src/
    ... existing core ...
    fd16_frame.c
    phy/
      phy_lvds_fd16.c
    ... existing PHYs remain intact ...

Exact file paths must be verified against the checked-out repository. If the current tree differs, adapt to its actual abstraction boundaries instead of forcing the paths above.

# 7. New FD16 frame API

#pragma once
#include <stdint.h>
#include <stdbool.h>
#include <stddef.h>

#define ABUS_FD16_CHANNELS       16
#define ABUS_FD16_AUDIO_BYTES    (ABUS_FD16_CHANNELS * sizeof(int32_t))
#define ABUS_FD16_HEADER_BYTES   6
#define ABUS_FD16_CRC_BYTES      2
#define ABUS_FD16_FRAME_BYTES    72
#define ABUS_FD16_BITRATE        27648000UL
#define ABUS_FD16_CLOCK_HZ       27648000UL

uint16_t abus_fd16_crc16(const uint8_t *data, size_t len);
void abus_fd16_pack(uint8_t *frame, uint16_t sequence,
                    const int32_t samples[ABUS_FD16_CHANNELS]);
bool abus_fd16_unpack(const uint8_t *frame, uint16_t *sequence,
                      int32_t samples[ABUS_FD16_CHANNELS]);

_Static_assert(ABUS_FD16_FRAME_BYTES == 72,
               "FD16 frame must be 72 bytes");

# 8. PHY behavior

The new PHY is full-duplex and has no direction switching. Both drivers and receivers remain enabled. It must expose the core as two independent streams: one 72-byte TX frame every 20.833333 us and one 72-byte RX frame every 20.833333 us.

FD16 PHY responsibilities:
  - initialize SN65LVDS049 GPIO interface
  - configure PARLIO TX for 1-bit data
  - configure PARLIO RX for 1-bit data
  - use the 27.648 MHz clock as the bit clock
  - manage fixed-size 72-byte DMA transactions
  - report TX/RX/CRC/sequence statistics
  - never perform turnaround or high-Z direction switching
  - never use 8b10b in FD16 mode

# 9. ESP32-P4 / PARLIO clocking

ESP-IDF 5.5 documents external-clock mode for PARLIO TX and RX. TX can take an external clock through clk_in_gpio_num with PARLIO_CLK_SRC_EXTERNAL; RX has ext_clk_freq_hz, exp_clk_freq_hz and clk_in_gpio_num for external-clock operation, plus a free_clk flag for a free-running external clock. Verify the exact API available in the project's ESP-IDF revision before coding.

ESP-IDF P4 PARLIO TX external clock documentation: https://docs.espressif.com/projects/esp-idf/en/v5.5/esp32p4/api-reference/peripherals/parlio/parlio_tx.html

ESP-IDF P4 PARLIO RX external clock documentation: https://docs.espressif.com/projects/esp-idf/en/v5.5/esp32p4/api-reference/peripherals/parlio/parlio_rx.html

ESP-IDF P4 clock tree: https://docs.espressif.com/projects/esp-idf/en/v5.5/esp32p4/api-reference/peripherals/clk_tree.html

Recommended implementation: use a Si5351A on node A as the 27.648 MHz transport-clock source, feed that clock to the local SN65LVDS049 LVDS driver and to local PARLIO as its external clock, then receive the same LVDS clock at node B and use it as B's external PARLIO clock. Reuse any existing Si5351A infrastructure only after checking its current API and accuracy.

# 10. SN65LVDS049 electrical design constraints

TI specifies the SN65LVDS049 as a dual LVDS transceiver with two transmitters and two receivers, 3.3 V supply, up to 400 Mbps signaling, and intended point-to-point transmission over media of approximately 100 ohms. TI explicitly notes that ultimate distance/rate depends on media attenuation, coupled noise, and application-specific conditions. Therefore 50 m at 27.648 Mb/s is a design target to validate, not a guaranteed distance claim.

TI SN65LVDS049 product page: https://www.ti.com/product/SN65LVDS049

TI SN65LVDS049 datasheet: https://www.ti.com/lit/ds/symlink/sn65lvds049.pdf

Implementation rules:

Use 100-ohm controlled-impedance differential routing for the LVDS paths.

Place the 100-ohm receiver termination physically close to the receiving SN65LVDS049 pins.

Keep the PCB differential pairs short, symmetric, and referenced to a continuous ground plane.

Minimize vias and stubs between the SN65LVDS049 and the cable connector.

Use low-capacitance ESD protection at the external cable interface only after checking its effect on the eye opening.

Do not treat the Cat6A shield as a substitute for the DC common-mode reference.

Verify the A/B ground offset in the actual power architecture; the LVDS receiver common-mode range is finite.

# 11. Connector and cable assignment

| Pair | Signal | Direction | Notes |
| --- | --- | --- | --- |
| 1 | DATA A->B | A to B | 100-ohm LVDS, receiver termination at B |
| 2 | DATA B->A | B to A | 100-ohm LVDS, receiver termination at A |
| 3 | CLOCK A->B | A to B | 27.648 MHz LVDS, receiver termination at B |
| 4 | REF | bidirectional reference strategy | No power; define shield/chassis/0-V strategy explicitly |

# 12. Core integration rules

Do not retrofit the existing half-duplex state machine into FD16. Add an explicit FD16 execution path. The existing AudioBus page describes a half-duplex frame with downstream and upstream sections plus a guard interval; that architecture remains valid for the legacy modes but is not the FD16 transport model.

Legacy PHYs:
  keep current framing, encoding, direction switching and timing.

FD16:
  TX frame and RX frame proceed simultaneously.
  No discovery frame is required.
  No slot-map negotiation is required.
  No turnaround guard is required.
  No 8b10b codec is required.

# 13. No dynamic discovery in FD16

FD16 is a fixed point-to-point product profile. The endpoint knows at compile time that it is operating with 16 channels, 48 kHz and 32-bit samples. Do not force the existing multi-node discovery/slot-map protocol into the 72-byte FD16 frame.

# 14. Error handling

| Condition | Action |
| --- | --- |
| CRC failure | Drop current frame; output the last valid frame/sample set; increment counter. |
| Wrong sync/format | Drop frame; increment sync/format counter. |
| Unexpected sequence | Accept frame but increment sequence-error counter; do not retransmit. |
| Several consecutive bad frames | After a configurable threshold, ramp/mute output safely. |
| TX underrun | Report counter; preserve continuous clock. |
| RX overrun | Report counter; recover next fixed-size frame boundary. |

# 15. Statistics and diagnostics

uint32_t fd16_rx_frames;
uint32_t fd16_tx_frames;
uint32_t fd16_crc_errors;
uint32_t fd16_sync_errors;
uint32_t fd16_sequence_errors;
uint32_t fd16_rx_overruns;
uint32_t fd16_tx_underruns;
uint32_t fd16_consecutive_errors;
uint16_t fd16_last_seq;

Expose these counters through the existing AudioBus statistics interface if the current API makes that possible without breaking ABI. Otherwise add an FD16-specific statistics getter.

# 16. DMA and ISR rules

Use fixed-size DMA buffers; target two TX and two RX buffers of 72 bytes each initially.

No malloc/calloc/free in ISR/callback context.

Callbacks should only validate the DMA transaction, copy/queue samples, update counters and wake a task if needed.

Keep the clock free-running; do not gate it every audio frame unless a later measurement proves it necessary.

# 17. Wire byte order

Serialize int32_t audio samples explicitly, most-significant byte first. Do not memcpy native int32_t values directly onto the wire unless the implementation deliberately documents and enforces the CPU endianness. The same rule applies to the 16-bit sequence and CRC fields.

# 18. Proposed pin-level organization

Node A
  ESP32-P4 PARLIO TX data      -> SN65LVDS049 DIN1
  ESP32-P4 PARLIO RX data      <- SN65LVDS049 ROUT1
  Si5351A 27.648 MHz           -> SN65LVDS049 DIN2
  SN65LVDS049 DOUT1            -> CAT6A Pair 1
  SN65LVDS049 DOUT2            -> CAT6A Pair 3
  CAT6A Pair 2                 -> SN65LVDS049 RIN1

Node B
  ESP32-P4 PARLIO TX data      -> SN65LVDS049 DIN1
  ESP32-P4 PARLIO RX data      <- SN65LVDS049 ROUT1
  CAT6A Pair 3                 -> SN65LVDS049 RIN2 -> external PARLIO clock
  SN65LVDS049 DOUT1            -> CAT6A Pair 2
  CAT6A Pair 1                 -> SN65LVDS049 RIN1

The exact ESP32-P4 GPIO numbers are intentionally not fixed here. Cline must inspect the target board/schematic configuration and check for GPIO conflicts before selecting them.

# 19. Validation plan - software first

Unit-test CRC with known vectors.

Round-trip pack/unpack for signed min/max, zero, alternating patterns and random int32_t samples.

Verify exactly 72 bytes per frame.

Verify sequence wrap from 0xFFFF to 0x0000.

Inject one-bit and multi-bit frame errors and verify CRC rejection.

Verify one-frame repeat-last-sample concealment behavior.

Build legacy AudioBus configurations to prove the new FD16 code did not break existing modes.

# 20. Validation plan - electrical / link

Do not accept “audio works” as proof of link margin. First run a deterministic PRBS/BERT-style test. The engineering target is BER <= 1e-12 at 50 m. This is a validation target, not a vendor guarantee for the SN65LVDS049 at 50 m.

| Distance | Test | Pass criterion |
| --- | --- | --- |
| 1 m | PRBS31, both directions | BER <= 1e-12 |
| 10 m | PRBS31, both directions simultaneously | BER <= 1e-12 |
| 25 m | PRBS31 + eye diagram | BER <= 1e-12, adequate eye opening |
| 35 m | PRBS31 + eye diagram | BER <= 1e-12 |
| 50 m | PRBS31 + eye + common-mode | BER <= 1e-12 |
| 50 m / 24 h | continuous bidirectional stress | No unexplained loss of sync or CRC burst |

# 21. Validation plan - temperature and real environment

Test approximately -10 C, +25 C and +50 C as an engineering characterization set.

Test several physical Cat6A cables/brands and the final intended connector/patch-panel path.

Test with the two directions active simultaneously.

Test near realistic EMI sources used by the product.

Record CRC errors, sequence gaps, eye opening and common-mode voltage.

# 22. Acceptance criteria for the code change

| Area | Acceptance criterion |
| --- | --- |
| Build | Project compiles with FD16 enabled using the repository’s actual ESP-IDF version. |
| Regression | Existing supported PHY configurations still compile and tests pass. |
| Framing | 72-byte fixed frame, 27.648 Mb/s, 48 kHz exact relation. |
| Duplex | TX and RX operate continuously and simultaneously. |
| Clock | A is sole master; B uses received external LVDS clock. |
| Errors | CRC/sequence/error counters observable; no retransmission protocol. |
| Memory | No dynamic allocation in ISR/callback path. |
| Tests | Unit tests + integration/link tests included or scripted. |
| Documentation | Public configuration and hardware assumptions documented in the repo. |

# 23. Cline execution plan to generate

Cline should produce a plan with these phases, then implement them in order. The exact files are to be discovered from the checked-out tree rather than assumed.

Phase 0 - Repository audit: tree, component boundaries, Kconfig, CMake, current PHY interface, AudioBus core state machine, PARLIO helpers, tests, board pin definitions, ESP-IDF version.

Phase 1 - Frame module: fd16_frame.h/c, unit tests, CRC and serialization.

Phase 2 - PHY interface: add FD16 PHY abstraction without changing legacy PHY behavior.

Phase 3 - PARLIO/DMA: 1-bit TX/RX, 72-byte transfers, 27.648 MHz external clocking, ISR-safe callbacks.

Phase 4 - Clock source: integrate/reuse Si5351A only if present and verify exact 27.648 MHz generation.

Phase 5 - Audio core: fixed 16-channel scheduling and simultaneous TX/RX.

Phase 6 - Diagnostics: counters, sequence checks, CRC error handling, repeat-last-sample.

Phase 7 - Tests: unit, integration, regression, stress.

Phase 8 - Documentation: hardware pin map, cable map, timing, configuration, known limitations.

# 24. Critical implementation cautions

Do not claim 50 m is guaranteed by the SN65LVDS049. TI states that achievable distance/rate depends on attenuation, noise and application-specific conditions.

Do not use the SN65LVDS049 as if it were a SerDes with CDR; it is a dual LVDS driver/receiver pair.

Do not leave the transport clock implicit. B must receive and use the dedicated LVDS clock.

Do not couple the FD16 mode to the legacy half-duplex guard/turnaround state machine.

Do not add 8b10b to FD16 unless a future requirement explicitly justifies the higher line rate and the clocking design is revisited.

Do not use unverified ESP-IDF field names from older/newer releases; compile against the project’s actual version.

Do not assume the current repository content is identical to the examples in this brief; inspect before patching.

# 25. Useful source references

AudioBus repository: https://github.com/DatanoiseTV/AudioBus

TI SN65LVDS049 product page: https://www.ti.com/product/SN65LVDS049

TI SN65LVDS049 datasheet: https://www.ti.com/lit/ds/symlink/sn65lvds049.pdf

ESP-IDF 5.5 ESP32-P4 PARLIO TX: https://docs.espressif.com/projects/esp-idf/en/v5.5/esp32p4/api-reference/peripherals/parlio/parlio_tx.html

ESP-IDF 5.5 ESP32-P4 PARLIO RX: https://docs.espressif.com/projects/esp-idf/en/v5.5/esp32p4/api-reference/peripherals/parlio/parlio_rx.html

ESP-IDF 5.5 ESP32-P4 clock tree: https://docs.espressif.com/projects/esp-idf/en/v5.5/esp32p4/api-reference/peripherals/clk_tree.html

# 26. Paste-ready prompt for Cline

Use the following text as the agent instruction after attaching this document to the workspace context:

You are modifying the checked-out DatanoiseTV/AudioBus repository. Use the attached AudioBus FD16 Implementation Brief as the design specification.

First: inspect the repository and produce a concrete implementation plan. Identify the actual files/interfaces that correspond to the brief; do not assume file names or ESP-IDF APIs. Verify the project’s ESP-IDF version, current PHY abstraction, PARLIO helpers, Kconfig/CMake, board GPIO definitions, and existing tests.

Target: add a new fixed point-to-point full-duplex FD16 profile:
- 16 audio channels in each direction
- 32-bit signed samples
- 48 kHz
- 72-byte fixed frame per sample
- 27.648 Mb/s NRZ transport
- 27.648 MHz dedicated LVDS clock from A to B
- 1 x SN65LVDS049 per endpoint
- Cat6A, four pairs, target 50 m
- no 8b10b, no CDR, no turnaround, no half-duplex direction switching
- CRC-16-CCITT + sequence counter + repeat-last-sample error concealment
- fixed point-to-point operation; no discovery/slot-map negotiation in FD16

Implementation rules:
1. Preserve all legacy AudioBus PHYs and their existing behavior.
2. Add the FD16 path as a separate profile/PHY.
3. Use fixed-size DMA buffers and ISR-safe callbacks; no heap allocation in ISR context.
4. Use the ESP-IDF APIs that actually exist in the checked-out project version.
5. Do not invent GPIO numbers; derive them from the project/board configuration.
6. Verify exact PARLIO external-clock configuration for both TX and RX.
7. Keep 27.648 MHz / 576 bits = 48 kHz as an invariant.
8. Add unit tests for packing, CRC, sequence wrap and corruption.
9. Add integration/regression coverage sufficient to prove legacy builds are unaffected.
10. Document hardware assumptions, termination, cable pair assignment and the 50 m link validation strategy.

Before coding, return a plan with: files to edit/create, interface changes, data flow, state machine, clock tree, DMA configuration, test strategy, and risks. After approval/plan consistency, implement in small steps and compile/test after each significant change.

End of implementation brief